# Lab3: Internet Connectivity Using ESP32

## 1. Lab overview

Aim: Connect an ESP32 to the Internet over Wi-Fi, then use that link to serve a local web page, push sensor data to a cloud dashboard, all over HTTP.

Duration: 3 hours (one lab session), split into three parts of about 45–60 minutes each.

Class size: 100 students in 50 pairs, one kit per pair, in one lab room. The lab uses a shared, instructor-run network (section 2), so students stay at their benches throughout.

Learning objectives. By the end of the lab, students can:

- Explain the difference between ESP32 station (STA) and access point (AP) modes.

- Scan for networks, join a Wi-Fi network and read the assigned IP address, gateway and RSSI.

- Host an HTTP web server on the ESP32 that reads a sensor and switches an LED from a browser.

- Send sensor readings to a cloud service using an HTTP GET/POST request.

- Explain HTTP methods and status codes, and the difference between the ESP32 as an HTTP server and as an HTTP client.

- Diagnose common connectivity faults (wrong credentials, weak signal, 5 GHz network, no Internet access).

Prerequisites: Basic Arduino C/C++ (setup/loop, digitalWrite, Serial), a breadboard wiring lab, and an introduction to IP addressing.

Lab flow

## 2. Hardware and software requirements

One kit per pair: 50 kits for 100 students, plus 2–3 spare kits.

| Item | Qty | Notes |
| --- | --- | --- |
| ESP32 DevKit V1 (ESP-WROOM-32, 30 or 38 pin) | 1 | Any ESP32 board with USB-serial works |
| Micro-USB or USB-C data cable | 1 | Must be a data cable, not charge-only |
| DHT11 or DHT22 temperature/humidity sensor | 1 | 3-pin module has the pull-up built in |
| 5 mm LED | 1 | Any colour |
| 220 Ω resistor | 1 | LED current limiter |
| 10 kΩ resistor | 1 | Only for a bare 4-pin DHT sensor |
| Push button (tactile switch) | 1 | Optional, used in extension tasks |
| Breadboard + jumper wires | 1 set | Male-to-male |
| Lab Wi-Fi (2.4 GHz), set up by the instructor | 1 | See network setup below; ESP32 does not support 5 GHz |
| Laptop/PC | 1 | Windows, macOS or Linux |

Software setup

- Install Arduino IDE 2.x.

- In File → Preferences → Additional boards manager URLs, add https://espressif.github.io/arduino-esp32/package_esp32_index.json.

- In Boards Manager, install esp32 by Espressif Systems. This includes the WiFi, WebServer and HTTPClient libraries.

- In Library Manager, install DHT sensor library (Adafruit) and Adafruit Unified Sensor.

- Select Tools → Board → ESP32 Dev Module and the correct COM/serial port. Set Serial Monitor to 115200 baud.

- If the port does not appear, install the CP210x or CH340 USB-UART driver for the board.

For 100 students, preinstall all of this on the lab PCs before the session.

Accounts and tools

- A free ThingSpeak account per pair, created before the lab (Part C).

Lab network setup for 100 students (instructor, before the lab)

A single home-style router cannot hold 50 ESP32s plus 50 lab PCs. Fifty phone hotspots in one room would also jam the 2.4 GHz band. Set up one shared network instead:

| Item | Recommendation |
| --- | --- |
| Access points | 3 APs (or one controller with 3 radios), 17–20 ESP32s per AP |
| Channels | Fixed 2.4 GHz channels 1, 6 and 11 (non-overlapping) |
| SSIDs | IOTLAB-1, IOTLAB-2, IOTLAB-3, WPA2-PSK, assigned by bench row |
| Band | 2.4 GHz only on these SSIDs; band steering off |
| DHCP pool | At least 150 addresses (e.g. 192.168.10.50–250), 2 h lease |
| Client isolation | Off, so lab PCs can open ESP32 web pages (Part B) |
| Lab PCs | On the same subnet (wired is best); students use these, not phones |
| Internet uplink | Needed for Part C only; the firewall/proxy must allow api.thingspeak.com on port 80. 50 small requests every 20 s is negligible bandwidth |

Class rules

- Each pair gets a group ID (grp01–grp50) and an assigned SSID, printed on a bench card. Name the ThingSpeak channel with the group ID, e.g. ESP32 Lab grp07.

- Phones stay off the lab Wi-Fi, and phone hotspots stay off for the whole lab.

- Power boards up row by row when the instructor calls it, so 50 boards don't all ask DHCP for an address at the same moment.

- Each pair creates its ThingSpeak account as homework before the lab, because sign-up needs email verification.

- Plan for about 1 teaching assistant per 12–15 pairs to sign off checkpoints.

## 3. Background theory

The ESP32 is a dual-core 240 MHz microcontroller with built-in 2.4 GHz Wi-Fi (802.11 b/g/n) and Bluetooth, so it can join a network and speak TCP/IP without any extra module.

Wi-Fi modes

| Mode | What the ESP32 does | Typical use |
| --- | --- | --- |
| Station (WIFI_STA) | Joins an existing router or hotspot and gets an IP by DHCP | Sending data to the Internet |
| Access point (WIFI_AP) | Creates its own network (default IP 192.168.4.1) | Setup pages, offline control |
| AP + STA (WIFI_AP_STA) | Both at once | Provisioning, bridging |

Key networking terms

- SSID — the network name. RSSI — received signal strength in dBm; −50 is excellent, −70 is fair, below −80 is unreliable.

- DHCP — the access point or router assigns the ESP32 an IP address, subnet mask, gateway and DNS server.

- Local IP vs public IP — the ESP32 gets a private address (e.g. 192.168.10.x). Devices on the same network can reach it; the Internet cannot.

- HTTP — request/response over TCP port 80 (443 for HTTPS). The client sends a method (GET to read, POST to send data) to a URL. The server replies with a status code (200 OK, 303 redirect, 404 not found) and a body (HTML, JSON or text).

- Server vs client — in Part B the ESP32 is the server and the browser asks it for pages. In Part C the ESP32 is the client and ThingSpeak is the server.

How data travels in this lab

Part B stays inside the lab network. Part C goes out to the Internet, so anyone can view the data.

## 4. Circuit setup

Wire the circuit once; all four parts use the same connections.

| Component | Component pin | ESP32 pin | Notes |
| --- | --- | --- | --- |
| LED | Anode (long leg) via 220 Ω | GPIO 2 | GPIO 2 is also the on-board blue LED on most DevKits |
| LED | Cathode (short leg) | GND |  |
| DHT11/DHT22 | VCC | 3V3 | Do not use 5 V/VIN on bare sensors |
| DHT11/DHT22 | DATA | GPIO 4 | 10 kΩ pull-up to 3V3 if sensor is bare 4-pin |
| DHT11/DHT22 | GND | GND |  |
| Push button | One side | GPIO 15 | Use INPUT_PULLUP in code |
| Push button | Other side | GND |  |

Pins to avoid: GPIO 6–11 (connected to on-board flash), and GPIO 34–39 as outputs (input-only). ADC2 pins (GPIO 0, 2, 4, 12–15, 25–27) cannot do analogRead() while Wi-Fi is on — use ADC1 pins (GPIO 32–39) for any analog sensor.

Check before powering: instructor verifies no short between 3V3 and GND, LED polarity, and sensor pin order.

## 5. Part A — Scan and connect to Wi-Fi (station mode)

Goal: list nearby networks, join one, and print the network details the router assigned.

Procedure

- Replace YOUR_SSID and YOUR_PASSWORD with the SSID on your bench card (e.g. IOTLAB-2) and the lab password.

- Upload the sketch when your row is called, open Serial Monitor at 115200 baud, and press the board's EN button.

- In the scan output, find the three lab SSIDs. Record each one's channel and RSSI at your bench, and count how many other networks share that channel (Table A).

- Record your IP, gateway, MAC and time to connect.

- Signal experiment at the bench: watch the RSSI printed every 2 s while you (a) leave the antenna clear, (b) cover it with your hand, (c) place the board inside a metal tin or wrap it in aluminium foil, (d) put it behind the laptop screen. Record each value.

- Hand your bench RSSI to the TA. The instructor combines all 50 values into a room signal map and discusses why benches differ.

#include <WiFi.h>

const char* ssid     = "YOUR_SSID";
const char* password = "YOUR_PASSWORD";

void scanNetworks() {
  Serial.println("Scanning...");
  int n = WiFi.scanNetworks();
  for (int i = 0; i < n; i++) {
    Serial.printf("%2d: %-24s RSSI %4d dBm  CH %2d  %s\n",
      i + 1, WiFi.SSID(i).c_str(), WiFi.RSSI(i), WiFi.channel(i),
      WiFi.encryptionType(i) == WIFI_AUTH_OPEN ? "open" : "secured");
  }
  WiFi.scanDelete();
}

void connectWiFi() {
  WiFi.begin(ssid, password);
  Serial.print("Connecting");
  unsigned long start = millis();
  while (WiFi.status() != WL_CONNECTED && millis() - start < 20000) {
    delay(500);
    Serial.print(".");
  }
  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("\nConnected!");
    Serial.print("IP address : "); Serial.println(WiFi.localIP());
    Serial.print("Gateway    : "); Serial.println(WiFi.gatewayIP());
    Serial.print("Subnet     : "); Serial.println(WiFi.subnetMask());
    Serial.print("DNS        : "); Serial.println(WiFi.dnsIP());
    Serial.print("MAC        : "); Serial.println(WiFi.macAddress());
    Serial.print("RSSI (dBm) : "); Serial.println(WiFi.RSSI());
  } else {
    Serial.printf("\nFailed, status code %d\n", WiFi.status());
  }
}

void setup() {
  Serial.begin(115200);
  WiFi.mode(WIFI_STA);
  WiFi.disconnect();
  delay(100);
  scanNetworks();
  connectWiFi();
}

void loop() {
  Serial.printf("RSSI now: %d dBm\n", WiFi.RSSI());   // used in step 5
  delay(2000);
  // Auto-reconnect if the link drops
  if (WiFi.status() != WL_CONNECTED) {
    Serial.println("Wi-Fi lost, reconnecting...");
    WiFi.reconnect();
    delay(5000);
  }
}

Expected output (example)

Scanning...
 1: LabNet-2G                RSSI  -48 dBm  CH  6  secured
 2: Pesu-Hotspot             RSSI  -61 dBm  CH 11  secured
Connecting.....
Connected!
IP address : 192.168.1.37
Gateway    : 192.168.1.1
...

Checkpoint A: show the instructor the IP address and explain which device assigned it.

## 6. Part B — ESP32 web server (LED control + live readings)

Goal: make the ESP32 an HTTP server. A lab PC on the same network opens the ESP32's IP in a browser, sees temperature and humidity, and turns the LED on or off.

Procedure

- Upload the sketch below with the SSID and password from your bench card. Note the IP printed in Serial Monitor.

- On your lab PC (on the lab network), open http://<ESP32-IP>/.

- Click ON / OFF and confirm the LED responds.

- Try the raw endpoints /led/on, /led/off, /data and /xyz directly in the address bar and record the responses (Table B).

- Measure response time: open the browser's developer tools (Network tab) and note the status code and time for each request.

- With the neighbouring pair's permission, open their ESP32's IP and switch their LED. Note what this shows about security on a shared network.

#include <WiFi.h>
#include <WebServer.h>
#include <DHT.h>

const char* ssid     = "YOUR_SSID";
const char* password = "YOUR_PASSWORD";

#define LED_PIN  2
#define DHT_PIN  4
#define DHT_TYPE DHT11   // change to DHT22 if needed

DHT dht(DHT_PIN, DHT_TYPE);
WebServer server(80);
bool ledOn = false;

String page() {
  float t = dht.readTemperature();
  float h = dht.readHumidity();
  String s = "<!DOCTYPE html><html><head><meta name='viewport' content='width=device-width'>";
  s += "<meta http-equiv='refresh' content='5'><title>ESP32 Lab</title></head><body style='font-family:sans-serif;text-align:center'>";
  s += "<h2>ESP32 Web Server</h2>";
  s += "<p>Temperature: " + (isnan(t) ? String("--") : String(t, 1)) + " &deg;C</p>";
  s += "<p>Humidity: "    + (isnan(h) ? String("--") : String(h, 1)) + " %</p>";
  s += "<p>LED is <b>" + String(ledOn ? "ON" : "OFF") + "</b></p>";
  s += "<a href='/led/on'><button>ON</button></a> <a href='/led/off'><button>OFF</button></a>";
  s += "</body></html>";
  return s;
}

void handleRoot()   { server.send(200, "text/html", page()); }
void handleLedOn()  { ledOn = true;  digitalWrite(LED_PIN, HIGH); server.sendHeader("Location", "/"); server.send(303); }
void handleLedOff() { ledOn = false; digitalWrite(LED_PIN, LOW);  server.sendHeader("Location", "/"); server.send(303); }
void handleData() {
  String json = "{\"temperature\":" + String(dht.readTemperature(), 1) +
                ",\"humidity\":"    + String(dht.readHumidity(), 1) +
                ",\"led\":"         + String(ledOn ? "true" : "false") + "}";
  server.send(200, "application/json", json);
}
void handleNotFound() { server.send(404, "text/plain", "Not found"); }

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  dht.begin();
  WiFi.mode(WIFI_STA);
  WiFi.begin(ssid, password);
  while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }
  Serial.print("\nOpen http://"); Serial.println(WiFi.localIP());

  server.on("/", handleRoot);
  server.on("/led/on", handleLedOn);
  server.on("/led/off", handleLedOff);
  server.on("/data", handleData);
  server.onNotFound(handleNotFound);
  server.begin();
}

void loop() {
  server.handleClient();
}

Variation (AP mode) — instructor demo only: 50 boards each creating their own network would flood the 2.4 GHz band, so the instructor shows this once. Replace the Wi-Fi block with WiFi.softAP("ESP32-DEMO", "12345678"); and browse to http://192.168.4.1 after joining that network. The page works with no router or Internet.

Checkpoint B: control the LED from your lab PC's browser, show the JSON from /data, and explain why a phone on mobile data cannot open the same URL.

## 7. Part C — Send sensor data to the cloud over HTTP (ThingSpeak)

Goal: the ESP32 acts as an HTTP client, pushing temperature and humidity to a ThingSpeak channel every 20 seconds, where they plot as live charts viewable from anywhere.

Procedure

- Log in to ThingSpeak → Channels → New Channel. Name it ESP32 Lab, enable Field 1 = Temperature and Field 2 = Humidity, save.

- Open the API Keys tab and copy the Write API Key.

- Paste the key into the sketch, upload, and watch Serial Monitor for the HTTP response code.

- Open the channel's Private View and confirm points appear. Breathe on the sensor and watch the humidity chart rise.

- Make the channel public and open it on a phone over mobile data — compare with Checkpoint B.

#include <WiFi.h>
#include <HTTPClient.h>
#include <DHT.h>

const char* ssid     = "YOUR_SSID";
const char* password = "YOUR_PASSWORD";
const char* apiKey   = "YOUR_WRITE_API_KEY";
const char* server   = "http://api.thingspeak.com/update";

#define DHT_PIN  4
#define DHT_TYPE DHT11
DHT dht(DHT_PIN, DHT_TYPE);

const unsigned long INTERVAL_MS = 20000;  // free ThingSpeak accounts accept at most one update per 15 s
unsigned long lastSend = 0;

void setup() {
  Serial.begin(115200);
  dht.begin();
  WiFi.mode(WIFI_STA);
  WiFi.begin(ssid, password);
  while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }
  Serial.println("\nWi-Fi connected");
}

void loop() {
  if (millis() - lastSend < INTERVAL_MS) return;
  lastSend = millis();

  float t = dht.readTemperature();
  float h = dht.readHumidity();
  if (isnan(t) || isnan(h)) { Serial.println("Sensor read failed"); return; }

  if (WiFi.status() == WL_CONNECTED) {
    HTTPClient http;
    String url = String(server) + "?api_key=" + apiKey +
                 "&field1=" + String(t, 1) + "&field2=" + String(h, 1);
    http.begin(url);
    int code = http.GET();
    Serial.printf("T=%.1f C  H=%.1f %%  -> HTTP %d, entry id %s\n",
                  t, h, code, http.getString().c_str());
    http.end();
  } else {
    WiFi.reconnect();
  }
}

Reading the response: HTTP 200 with a number (the entry ID) means success. HTTP 200 with 0 means ThingSpeak rejected the update — usually a wrong key or sending faster than every 15 s. A negative code (e.g. −1) means the ESP32 could not reach the server.

Extension: rewrite the request as an HTTP POST with http.addHeader("Content-Type", "application/x-www-form-urlencoded") and a body of api_key=...&field1=....

Checkpoint C: show at least 10 points on both charts and state the HTTP method, host and port used.

## 8. Observations, troubleshooting and safety

Table B — Web server (Part B)

| Request | HTTP status | Response time (ms) | LED state after |
| --- | --- | --- | --- |
| / |  |  |  |
| /led/on |  |  |  |
| /led/off |  |  |  |
| /data |  |  |  |
| /xyz |  |  |  |

Table C — Data sent to ThingSpeak (Part C)

| Reading no. | Time | Temperature (°C) | Humidity (%) | HTTP code | Entry ID |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

Table D — ESP32 as HTTP server vs HTTP client

| Question | Part B (server) | Part C (client) |
| --- | --- | --- |
| Who starts the request? |  |  |
| HTTP method used |  |  |
| Host and port |  |  |
| Reachable from a phone on mobile data? |  |  |
| Minimum update interval |  |  |
| Delay from sensor change to display (s) |  |  |

Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| No COM port in Arduino IDE | Charge-only cable or missing driver | Swap cable; install CP210x/CH340 driver |
| Upload fails: "Failed to connect to ESP32" | Board not in boot mode | Hold BOOT while upload starts |
| Stuck printing dots, never connects | Wrong SSID/password, or a 5 GHz network | Check spelling and case against your bench card |
| WiFi.status() returns 1 (WL_NO_SSID_AVAIL) | SSID typo, or that AP is down | Check the SSID against your bench card; confirm with the Part A scan; tell the TA |
| Connects then drops repeatedly | Overloaded AP, weak signal or USB power sag | Keep the antenna clear of metal and laptops; use a powered USB port; ask the TA to move your group to a less loaded SSID |
| Connected but IP is 0.0.0.0 | DHCP pool full | Instructor enlarges the pool or shortens the lease |
| Browser cannot open ESP32 IP | PC on a different subnet, or client isolation on | Use a lab PC on the lab network; instructor turns off client isolation |
| Sensor shows nan / -- | Wrong pin, DHT11/22 type mismatch, no pull-up | Check wiring and DHT_TYPE; wait 2 s between reads |
| ThingSpeak returns 0 | Wrong write key or updates under 15 s apart | Re-copy key; keep interval at 20 s |
| Part C prints HTTP −1 | No Internet from the lab network, or firewall blocks api.thingspeak.com | Instructor checks the uplink and firewall |
| Random resets / brownout message | Insufficient power when Wi-Fi transmits | Better cable/port; 10 µF capacitor across 3V3–GND |

Safety and good practice

- Work only with USB power (5 V). Unplug before rewiring.

- Never connect 5 V to a GPIO pin; ESP32 I/O is 3.3 V only.

- Keep Wi-Fi passwords and API keys out of shared code and screenshots; blank them before submitting.

- Use only your own group's ESP32 and ThingSpeak channel, except for the permitted step 6 in Part B.